
Guide for the Application of Radio Frequency Bands Used by All Kinds of Radio Transmission Equipment during the 16th Asian Games/Guangzhou 2010 Asian Para Games
	Equipment Type
	Typical Band
	Remark

	Fixed Link
	1427-1525Mz 
	Y

	
	3600-4200MHz 
	Y

	
	4400-5000MHz 
	Y

	
	5925-7110MHz 
	Y

	
	7125-8500MHz 
	Y

	
	10700-11700MHz 
	Y

	
	12750-13250MHz 
	Y

	
	14250-15350MHz 
	Y

	
	17700-19700MHz 
	Y

	
	21200-23600MHz 
	Y

	Microwave Mobile Link
	1920-2025MHz 
	Y

	
	2110-2200MHz 
	Y

	
	2300-2690MHz 
	Y

	
	3100-3700MHz 
	Y ∆

	
	4400-5000MHz 
	Y

	
	5925-7110MHz 
	Y

	
	7125-8500MHz 
	Y

	
	10700-11700MHz 
	Y

	
	12750-13250MHz 
	Y

	
	14300-15350MHz 
	Y

	
	17700-19700MHz 
	Y

	
	21200-23600MHz 
	Y

	Fixed Satellite Link
	C band: 4/6GHz 
	O

	
	Ku band: 11/14GHz, 12/14GHz 
	G 

	
	Ka band: 20GHz, 30GHz 
	G

	Satellite News Gathering
	C band: 4/6GHz 
	O

	
	Ku band: 11/14GHz, 12/14GHz 
	G 

	
	Ka band: 20GHz, 30GHz 
	G

	Land Mobile Radio Service System
	138-167MHz 
	Y ∆

	
	403-425MHz (406-406.1MHz excluded)
	O 

	
	450-470MHz 
	O

	Talk Back System
	138-167MHz 
	Y ∆

	
	403-425MHz (406-406.1MHz excluded)  
	O

	
	450-470MHz 
	O

	Handheld Radio
	138-167MHz 
	Y ∆

	
	403-425MHz (406-406.1MHz excluded)  
	O

	
	450-470MHz 
	O

	Cordless Camera
	1920-2025MHz 
	Y

	
	2110-2200MHz 
	Y

	
	2300-2690MHz 
	Y

	
	3300-4200MHz 
	Y

	
	4400-5000MHz 
	Y

	
	5925-7110MHz 
	Y

	
	7125-7725MHz 
	Y

	In Ear Monitor system
	470-566MHz 
	B

	
	566-606MHz 
	Y

	
	606-798MHz 
	B

	
	798-806MHz 
	Y

	
	840-851MHz 
	Y

	Interpretation Service
	75.4-76MHz 
	G

	
	84-87MHz 
	Y

	Telemetry and Telecommand

	41-41.2MHz 
	G

	
	138-167MHz 
	Y ∆

	
	223-235MHz 
	Y

	
	450-470 MHz 
	O

	Wireless Microphone
	470-566MHz 
	B

	
	566-606MHz 
	Y

	
	606-798MHz 
	B

	
	798-806MHz 
	Y

	
	840-851MHz 
	Y

	Wireless LAN
	2400-2483.5MHz 
	Y

	
	5725-5850MHz 
	G

	Notes:
	
	

	
	1. In the form,

	Ｇ
	Green, means only a few users;
	
	

	Ｙ
	Yellow, means there are some current users;
	
	

	Ｏ
	Orange, means there are many current users and there is a rather high risk of interference;
	

	Ｂ
	Blue, means it can only be used after the coordination with the related authorities and other functional areas; 
	

	∆ 
	Means the band (or part of the band) is not in conformity with the provisions of China on frequency allocation, and its temporary use may be applied for during the 16th Asian Games/Guangzhou 2010 Asian Para Games and its preparation.
	

	2. It is suggested that the channel spacing for the special-purpose interphone system, land mobile radio communication business and portable interphone should be 12.5KHz;

	3. Where the applied frequency is not included in the above form, the advance communication with the General Office of the Joint Meeting for Radio Security for the 16th Asian Games is necessary for the negotiation and settlement.


